LINE

WALL COVER

Descriptive memories

| MODULE A LINE WALL COVER | M2. Supply and installation of LINE wall cover with hidden profiling, module A: totally blind. Standard module width: 1,200 mm. Total thickness: 36 mm. Covers walls, columns and vertical ducts. Telescopic system to compensate for uneven wall surfaces up to 118 mm. Made with Sendzimir galvanised-steel frame per EN 10.346: 2009 (DX51D+Z). Previously degreased visible metal components electrostatically painted with polyester or epoxy powder and polymerised at 200ºC. Total paint thickness of 50/90 micrometres and colour tolerance per DIN 5033. Compatible with LINE partition. 16 mm thick particle-board panels edged and laminated with semi-rigid vinyl coatings. Other surface finish options include fine hardwoods or melamine and lacquered MDF panels. There is no groove between panels. The panels close over a 2 mm thick PVC joint which provides sound insulation. Upper and bottom runners leave 13 mm to the floor and the ceiling. Interior insulation with 20 mm thick, 90 Kg/m3 dense mineral fibre. Components for window, door and corner joins. Re-installation and interchangeability of modules. Quality assurance EN ISO 9001: 2008 certified by LLOYD’S. Includes all parts necessary for installation. Supplied and installed by manufacturer-approved distributor. |
| --- | --- |

| MODULE B LINE WALL COVER | M2. Supply and installation of LINE wall cover with hidden profiling, module B: totally blind, divided in 3 panels (grooves at 882 and 2,082 mm). Standard module width: 1,200 mm. Total thickness: 36 mm. Covers walls, columns and vertical ducts. Telescopic system to compensate for uneven wall surfaces up to 118 mm. Made with Sendzimir galvanised-steel frame per EN 10.346: 2009 (DX51D+Z). Previously degreased visible metal components electrostatically painted with polyester or epoxy powder and polymerised at 200ºC. Total paint thickness of 50/90 micrometres and colour tolerance per DIN 5033. Compatible with LINE partition. 16 mm thick particle-board panels edged and laminated with semi-rigid vinyl coatings. Other surface finish options include fine hardwoods or melamine and lacquered MDF panels. There is no groove between panels. The panels close over a 2 mm thick PVC joint which provides sound insulation. Upper and bottom runners leave 13 mm to the floor and the ceiling. Interior insulation with 20 mm thick, 90 Kg/m3 dense mineral fibre. Components for window, door and corner joins. Re-installation and interchangeability of modules. Quality assurance EN ISO 9001: 2008 certified by LLOYD’S. Includes all parts necessary for installation. Supplied and installed by manufacturer-approved distributor. |
| --- | --- |

| MODULE E LINE WALL COVER | M2. Supply and installation of LINE wall cover with hidden profiling, module E: totally blind, divided in 2 panels (groove at 882 mm). Standard module width: 1,200 mm. Total thickness: 36 mm. Covers walls, columns and vertical ducts. Telescopic system to compensate for uneven wall surfaces up to 118 mm. Made with Sendzimir galvanised-steel frame per EN 10.346: 2009 (DX51D+Z). Previously degreased visible metal components electrostatically painted with polyester or epoxy powder and polymerised at 200ºC. Total paint thickness of 50/90 micrometres and colour tolerance per DIN 5033. Compatible with LINE partition. 16 mm thick particle-board panels edged and laminated with semi-rigid vinyl coatings. Other surface finish options include fine hardwoods or melamine and lacquered MDF panels. There is no groove between panels. The panels close over a 2 mm thick PVC joint which provides sound insulation. Upper and bottom runners leave 13 mm to the floor and the ceiling. Interior insulation with 20 mm thick, 90 Kg/m3 dense mineral fibre. Components for window, door and corner joins. Re-installation and interchangeability of modules. Quality assurance EN ISO 9001: 2008 certified by LLOYD’S. Includes all parts necessary for installation. Supplied and installed by manufacturer-approved distributor. |
| --- | --- |

| MODULE F LINE WALL COVER | M2. Supply and installation of LINE wall cover with hidden profiling, module F: totally blind, divided in 2 panels (groove at 2,082 mm). Standard module width: 1,200 mm. Total thickness: 36 mm. Covers walls, columns and vertical ducts. Telescopic system to compensate for uneven wall surfaces up to 118 mm. Made with Sendzimir galvanised-steel frame per EN 10.346: 2009 (DX51D+Z). Previously degreased visible metal components electrostatically painted with polyester or epoxy powder and polymerised at 200ºC. Total paint thickness of 50/90 micrometres and colour tolerance per DIN 5033. Compatible with LINE partition. 16 mm thick particle-board panels edged and laminated with semi-rigid vinyl coatings. Other surface finish options include fine hardwoods or melamine and lacquered MDF panels. There is no groove between panels. The panels close over a 2 mm thick PVC joint which provides sound insulation. Upper and bottom runners leave 13 mm to the floor and the ceiling.. Interior insulation with 20-mm thick, 90 Kg/m3 dense mineral fibre. Components for window, door and corner joins. Re-installation and interchangeability of modules. Quality assurance EN ISO 9001: 2008 certified by LLOYD’S. Includes all parts necessary for installation. Supplied and installed by manufacturer-approved distributor. |
| --- | --- |