LINE

MOVABLE PARTITION

Descriptive memories

| ### LINE MOVABLE PARTITION | Supply and installation of stackable Movable Line partition by Dynamobel with mono-directional / multi-directional suspension. Consisting of independent, retractable modules which join together and move via pulleys along a track made of extruded aluminium (30 micrometre hardened alloy), with no track on the floor. Total partition thickness: 106 mm. Telescopic interior mechanism actuated by a crank, with neoprene parts for a perfect seal at the top and bottom with a closing force of up to 70 kg. Double internal structure: an internal steel frame into which the interior mechanisms and carrying blocks are built, providing the rigidity needed for movable elements; and a perfectly fitted aluminium perimeter frame with visible / hidden profiling and a perimeter acoustic seal which allows the modules to lock together vertically by means of a double tongue-and-groove joint with a magnetic fastening system and acoustic seals to achieve perfect thermal and sound insulation. 19-mm thick, edged panels made of chipboard faced with semi-rigid vinyl / chipboard faced with fine hardwood / chipboard faced with melamine / lacquered MDF. Door leaf with handle and lock in stainless steel housed on both faces and an opening of 900 x 2120 mm. 3-mm groove between modules and 25-mm groove with floor and ceiling. Interior insulation consisting of 68-mm thick, 30/40-kg/m3 dense mineral fibre. Sound insulation of 40 / 44 / 50 dB. The Line system meets all the sections and specifications of the UNE-41956 "REMOVABLE WALLS" standard. ISO 9001:2008 quality management system certified by LLOYD’S REGISTER QUALITY ASSURANCE. Includes all parts needed for installation. Supplied and installed by manufacturer-approved distributor. |
| --- | --- |