1:10

WALL COVER

Descriptive memories

| MODULE A 1:10 WALL COVER | M2. Supply and installation of 1:10 wall cover with hidden profiling, module A: totally blind. Standard module width: 1,200 mm. Total thickness: 33 mm. Covers walls, columns and vertical ducts. Telescopic system to compensate for uneven wall surfaces up to 118 mm. Made with Sendzimir galvanised-steel frame per EN 10.346: 2009 (DX51D+Z). Previously degreased visible metal components electrostatically painted with polyester or epoxy powder and polymerised at 200ºC. Total paint thickness of 50/60 micrometres and colour tolerance per DIN 5033. Compatible with 1:10 partition. 13mm-thick particle-board panels edged and laminated with semi-rigid vinyl coatings. Other surface finish options include fine hardwoods and lacquered MDF panels. The grooves the modulation features are 5 mm wide and 13 mm deep, and the panels close over a 1mm-thick PVC joint which provides sound insulation. Interior insulation with 20-mm thick, 90 Kg/m3 dense mineral fibre. Components for window, door and corner joins. Re-installation and interchangeability of modules. Quality assurance EN ISO 9001: 2008 certified by LLOYD’S. Includes all parts necessary for installation. Supplied and installed by manufacturer-approved distributor. |
| --- | --- |

| MODULE B 1:10 WALL COVER | M2. Supply and installation of 1:10 wall cover with hidden profiling, module B: totally blind, divided in 3 panels (grooves at 917.5 and 2117.5 mm). Standard module width: 1,200 mm. Total thickness: 33 mm. Covers walls, columns and vertical ducts. Telescopic system to compensate for uneven wall surfaces up to 118 mm. Made with Sendzimir galvanised-steel frame per EN 10.346: 2009 (DX51D+Z). Previously degreased visible metal components electrostatically painted with polyester or epoxy powder and polymerised at 200ºC. Total paint thickness of 50/60 micrometres and colour tolerance per DIN 5033. Compatible with 1:10 partition. 13mm-thick particle-board panels edged and laminated with semi-rigid vinyl coatings. Other surface finish options include fine hardwoods and lacquered MDF panels. The grooves the modulation features are 5 mm wide and 13 mm deep, and the panels close over a 1mm-thick PVC joint which provides sound insulation. Interior insulation with 20-mm thick, 90 Kg/m3 dense mineral fibre. Components for window, door and corner joins. Re-installation and interchangeability of modules. Quality assurance EN ISO 9001: 2008 certified by LLOYD’S. Includes all parts necessary for installation. Supplied and installed by manufacturer-approved distributor. |
| --- | --- |

| MODULE E 1:10 WALL COVER | M2. Supply and installation of 1:10 wall cover with hidden profiling, module E: totally blind, divided in 2 panels (groove at 917.5 mm). Standard module width: 1,200 mm. Total thickness: 33 mm. Covers walls, columns and vertical ducts. Telescopic system to compensate for uneven wall surfaces up to 118 mm. Made with Sendzimir galvanised-steel frame per EN 10.346: 2009 (DX51D+Z). Previously degreased visible metal components electrostatically painted with polyester or epoxy powder and polymerised at 200ºC. Total paint thickness of 50/60 micrometres and colour tolerance per DIN 5033. Compatible with 1:10 partition. 13mm-thick particle-board panels edged and laminated with semi-rigid vinyl coatings. Other surface finish options include fine hardwoods and lacquered MDF panels. The grooves the modulation features are 5 mm wide and 13 mm deep, and the panels close over a 1mm-thick PVC joint which provides sound insulation. Interior insulation with 20-mm thick, 90 Kg/m3 dense mineral fibre. Components for window, door and corner joins. Re-installation and interchangeability of modules. Quality assurance EN ISO 9001: 2008 certified by LLOYD’S. Includes all parts necessary for installation. Supplied and installed by manufacturer-approved distributor. |
| --- | --- |

| MODULE F 1:10 WALL COVER | M2. Supply and installation of 1:10 wall cover with hidden profiling, module F: totally blind, divided in 2 panels (groove at 2,117.5 mm). Standard module width: 1,200 mm. Total thickness: 33 mm. Covers walls, columns and vertical ducts. Telescopic system to compensate for uneven wall surfaces up to 118 mm. Made with Sendzimir galvanised-steel frame per EN 10.346: 2009 (DX51D+Z). Previously degreased visible metal components electrostatically painted with polyester or epoxy powder and polymerised at 200ºC. Total paint thickness of 50/60 micrometres and colour tolerance per DIN 5033. Compatible with 1:10 partition. 13mm-thick particle-board panels edged and laminated with semi-rigid vinyl coatings. Other surface finish options include fine hardwoods and lacquered MDF panels. The grooves the modulation features are 5 mm wide and 13 mm deep, and the panels close over a 1mm-thick PVC joint which provides sound insulation. Interior insulation with 20-mm thick, 90 Kg/m3 dense mineral fibre. Components for window, door and corner joins. Re-installation and interchangeability of modules. Quality assurance EN ISO 9001: 2008 certified by LLOYD’S. Includes all parts necessary for installation. Supplied and installed by manufacturer-approved distributor. |
| --- | --- |