1:10

SLIDING DOORS

Descriptive memories

| ### MODULE G Blind sliding door | M2. Supply and installation of 1:10 demountable partition with hidden profiling, module G: blind sliding door with floor-to-ceiling door jambs. Standard module width: 1,090 mm. Total thickness: 145 mm, with the door leaf gliding along a track fixed to the ceiling. Made with a self-supporting inner frame in Sendzimir galvanised steel and outer frame in 6060-T6 aluminium, per EN 10.346:2009 (DX51D+Z). Previously degreased visible metal components electrostatically painted with polyester or epoxy powder and polymerised at 200ºC. Paint thickness of 50/60 micrometres and colour tolerance per DIN 5033. Floor-to-ceiling door leaf made in particle-board panels laminated with semi-rigid vinyl coatings. Other surface finish options include fine hardwoods and lacquered MDF panels. Door leaf is 1,097 mm wide and 40 mm thick. Hardware: dampened rollers with stoppers in the track and vertical door handle. Re-installation and interchangeability of modules. Quality assurance EN ISO 9001:2008 certified by LLOYD’S. Includes all parts necessary for installation. Supplied and installed by manufacturer-approved distributor. |
| --- | --- |

| ### MODULE G Framed glazed sliding door | M2. Supply and installation of 1:10 demountable partition with hidden profiling, module G: framed glazed sliding door with floor-to-ceiling door jambs. Standard module width: 1,090 mm. Total thickness: 155 mm, with the door leaf gliding along a track fixed to the ceiling. Made with a self-supporting inner frame in Sendzimir galvanised steel and outer frame in 6060-T6 aluminium, per EN 10.346:2009 (DX51D+Z). Previously degreased visible metal components electrostatically painted with polyester or epoxy powder and polymerised at 200ºC. Paint thickness of 50/60 micrometres and colour tolerance per DIN 5033. Floor-to-ceiling single-glazed door leaf, with an 8mm-thick tempered glass pane framed by painted aluminium profiles, which is 1,097 mm wide and 50 mm thick. Hardware: dampened rollers with stoppers in the track and vertical door handle. Re-installation and interchangeability of modules. Quality assurance EN ISO 9001:2008 certified by LLOYD’S. Includes all parts necessary for installation. Supplied and installed by manufacturer-approved distributor. |
| --- | --- |

| ### MODULE G Clarit sliding door | M2. Supply and installation of 1:10 demountable partition with hidden profiling, module G: glazed sliding door with floor-to-ceiling door jambs. Standard module width: 1,090 mm. Total thickness: 126.5 mm, with the door leaf gliding along a track fixed to the ceiling. Made with a self-supporting inner frame in Sendzimir galvanised steel and outer frame in 6060-T6 aluminium, per EN 10.346:2009 (DX51D+Z). Previously degreased visible metal components electrostatically painted with polyester or epoxy powder and polymerised at 200ºC. Paint thickness of 50/60 micrometres and colour tolerance per DIN 5033. Floor-to-ceiling tempered glazed Clarit door leaf, 1,097 mm wide and 10 mm thick. Hardware: dampened rollers with stoppers in the track and vertical door handle. Re-installation and interchangeability of modules. Quality assurance EN ISO 9001:2008 certified by LLOYD’S. Includes all parts necessary for installation. Supplied and installed by manufacturer-approved distributor. |
| --- | --- |

| ### MODULE W Blind sliding door | M2. Supply and installation of 1:10 demountable partition with hidden profiling, module W: blind double sliding door with floor-to-ceiling door jambs. Standard module width: 2,190 mm. Total thickness: 145 mm, with the door leaves gliding along a track fixed to the ceiling. Made with a self-supporting inner frame in Sendzimir galvanised steel and outer frame in 6060-T6 aluminium, per EN 10.346:2009 (DX51D+Z). Previously degreased visible metal components electrostatically painted with polyester or epoxy powder and polymerised at 200ºC. Paint thickness of 50/60 micrometres and colour tolerance per DIN 5033. Floor-to-ceiling door leaves made in particle-board panels laminated with semi-rigid vinyl coatings. Other surface finish options include fine hardwoods and lacquered MDF panels. Door leaves are 1,097 mm wide and 40 mm thick. Hardware: dampened rollers with stoppers in the track and vertical door handles. Re-installation and interchangeability of modules. Quality assurance EN ISO 9001:2008 certified by LLOYD’S. Includes all parts necessary for installation. Supplied and installed by manufacturer-approved distributor. |
| --- | --- |

| ### MODULE W Framed glazed sliding door | M2. Supply and installation of 1:10 demountable partition with hidden profiling, module W: framed glazed double sliding door with floor-to-ceiling door jambs. Standard module width: 2,190 mm. Total thickness: 155 mm, with the door leaves gliding along a track fixed to the ceiling. Made with a self-supporting inner frame in Sendzimir galvanised steel and outer frame in 6060-T6 aluminium, per EN 10.346:2009 (DX51D+Z). Previously degreased visible metal components electrostatically painted with polyester or epoxy powder and polymerised at 200ºC. Paint thickness of 50/60 micrometres and colour tolerance per DIN 5033. Floor-to-ceiling single-glazed door leaves, with an 8mm-thick tempered glass panes framed by painted aluminium profiles, which are 1,097 mm wide and 50 mm thick. Hardware: dampened rollers with stoppers in the track and vertical door handles. Re-installation and interchangeability of modules. Quality assurance EN ISO 9001:2008 certified by LLOYD’S. Includes all parts necessary for installation. Supplied and installed by manufacturer-approved distributor. |
| --- | --- |

| ### MODULE W Clarit sliding door | M2. Supply and installation of 1:10 demountable partition with hidden profiling, module G: glazed sliding double door with floor-to-ceiling door jambs. Standard module width: 2,190 mm. Total thickness: 126.5 mm, with the door leaves gliding along a track fixed to the ceiling. Made with a self-supporting inner frame in Sendzimir galvanised steel and outer frame in 6060-T6 aluminium, per EN 10.346:2009 (DX51D+Z). Previously degreased visible metal components electrostatically painted with polyester or epoxy powder and polymerised at 200ºC. Paint thickness of 50/60 micrometres and colour tolerance per DIN 5033. Floor-to-ceiling tempered glazed Clarit door leaves, 1,097 mm wide and 10 mm thick. Hardware: dampened rollers with stoppers in the track and vertical door handles. Re-installation and interchangeability of modules. Quality assurance EN ISO 9001:2008 certified by LLOYD’S. Includes all parts necessary for installation. Supplied and installed by manufacturer-approved distributor. |
| --- | --- |