LINE

METALLIC WALL COVER

Descriptive memories

| MODULE A LINE METALLIC WALL COVER | M2. Supply and installation of LINE metallic wall cover with hidden profiling, module A: totally blind. Standard module width: 1,200 mm. Total thickness: 36 mm. Covers walls, columns and vertical ducts. Telescopic system to compensate for uneven wall surfaces up to 118 mm. Made with Sendzimir galvanised-steel frame per EN 10.346: 2009 (DX51D+Z). Previously degreased visible metal components electrostatically painted with polyester or epoxy powder and polymerised at 200ºC. Total paint thickness of 50/90 micrometres and colour tolerance per DIN 5033. Compatible with LINE partition. 16 mm thick metallic panels in Sendzimir galvanised steel with 15 mm thick inner plasterboard. There is no groove between panels. The panels close over a 2 mm thick PVC joint which provides sound insulation. Upper and bottom runners leave 13 mm to the floor and the ceiling. Interior insulation with 20 mm thick, 90 Kg/m3 dense mineral fibre. Components for window, door and corner joins. Re-installation and interchangeability of modules. Quality assurance EN ISO 9001: 2008 certified by LLOYD’S. Includes all parts necessary for installation. Supplied and installed by manufacturer-approved distributor. |
| --- | --- |

| MODULE B LINE METALLIC WALL COVER | M2. Supply and installation of LINE metallic wall cover with hidden profiling, module B: totally blind, divided in 3 panels (grooves at 882 and 2,082 mm). Standard module width: 1,200 mm. Total thickness: 36 mm. Covers walls, columns and vertical ducts. Telescopic system to compensate for uneven wall surfaces up to 118 mm. Made with Sendzimir galvanised-steel frame per EN 10.346: 2009 (DX51D+Z). Previously degreased visible metal components electrostatically painted with polyester or epoxy powder and polymerised at 200ºC. Total paint thickness of 50/90 micrometres and colour tolerance per DIN 5033. Compatible with LINE partition. 16 mm thick metallic panels in Sendzimir galvanised steel with 15 mm thick inner plasterboard. There is no groove between panels. The panels close over a 2 mm thick PVC joint which provides sound insulation. Upper and bottom runners leave 13 mm to the floor and the ceiling. Interior insulation with 20 mm thick, 90 Kg/m3 dense mineral fibre. Components for window, door and corner joins. Re-installation and interchangeability of modules. Quality assurance EN ISO 9001: 2008 certified by LLOYD’S. Includes all parts necessary for installation. Supplied and installed by manufacturer-approved distributor. |
| --- | --- |

| MODULE E LINE METALLIC WALL COVER | M2. Supply and installation of LINE metallic wall cover with hidden profiling, module E: totally blind, divided in 2 panels (groove at 882 mm). Standard module width: 1,200 mm. Total thickness: 36 mm. Covers walls, columns and vertical ducts. Telescopic system to compensate for uneven wall surfaces up to 118 mm. Made with Sendzimir galvanised-steel frame per EN 10.346: 2009 (DX51D+Z). Previously degreased visible metal components electrostatically painted with polyester or epoxy powder and polymerised at 200ºC. Total paint thickness of 50/90 micrometres and colour tolerance per DIN 5033. Compatible with LINE partition. 16 mm thick metallic panels in Sendzimir galvanised steel with 15 mm thick inner plasterboard. There is no groove between panels. The panels close over a 2 mm thick PVC joint which provides sound insulation. Upper and bottom runners leave 13 mm to the floor and the ceiling. Interior insulation with 20 mm thick, 90 Kg/m3 dense mineral fibre. Components for window, door and corner joins. Re-installation and interchangeability of modules. Quality assurance EN ISO 9001: 2008 certified by LLOYD’S. Includes all parts necessary for installation. Supplied and installed by manufacturer-approved distributor. |
| --- | --- |

| MODULE F LINE METALLIC WALL COVER | M2. Supply and installation of LINE metallic wall cover with hidden profiling, module F: totally blind, divided in 2 panels (groove at 2,082 mm). Standard module width: 1,200 mm. Total thickness: 36 mm. Covers walls, columns and vertical ducts. Telescopic system to compensate for uneven wall surfaces up to 118 mm. Made with Sendzimir galvanised-steel frame per EN 10.346: 2009 (DX51D+Z). Previously degreased visible metal components electrostatically painted with polyester or epoxy powder and polymerised at 200ºC. Total paint thickness of 50/90 micrometres and colour tolerance per DIN 5033. Compatible with LINE partition. 16 mm thick metallic panels in Sendzimir galvanised steel with 15 mm thick inner plasterboard. There is no groove between panels. The panels close over a 2 mm thick PVC joint which provides sound insulation. Upper and bottom runners leave 13 mm to the floor and the ceiling. Interior insulation with 20-mm thick, 90 Kg/m3 dense mineral fibre. Components for window, door and corner joins. Re-installation and interchangeability of modules. Quality assurance EN ISO 9001: 2008 certified by LLOYD’S. Includes all parts necessary for installation. Supplied and installed by manufacturer-approved distributor. |
| --- | --- |