LINE

SLIDING DOORS

Descriptive memories

| ### MODULE G Blind outer sliding door | M2. Supply and installation of LINE demountable partition with hidden profiling, module G: blind outer sliding door with floor-to-ceiling door jambs. Standard module width: 1,058 mm. Total thickness: 148 mm, with the door leaf gliding along a track fixed to the ceiling. Made with a self-supporting inner frame in Sendzimir galvanised steel and outer frame in 6060-T6 aluminium, per EN 10.346:2009 (DX51D+Z). Previously degreased visible metal components electrostatically painted with polyester or epoxy powder and polymerised at 200ºC. Paint thickness of 50/60 micrometres and colour tolerance per DIN 5033. Floor-to-ceiling door leaf made in particle-board panels laminated with semi-rigid vinyl coatings. Other surface finish options include fine hardwoods or melamine and lacquered MDF panels. Door leaf is 1,058 mm wide and 40 mm thick. Hardware: dampened rollers with stoppers in the track, vertical short handle or vertical long handle with lock. Re-installation and interchangeability of modules. Quality assurance EN ISO 9001:2008 certified by LLOYD’S. Includes all parts necessary for installation. Supplied and installed by manufacturer-approved distributor. |
| --- | --- |

| ### MODULE G Framed glazed outer sliding door | M2. Supply and installation of LINE demountable partition with hidden profiling, module G: framed glazed outer sliding door with floor-to-ceiling door jambs. Standard module width: 1,058 mm. Total thickness: 148 mm, with the door leaf gliding along a track fixed to the ceiling. Made with a self-supporting inner frame in Sendzimir galvanised steel and outer frame in 6060-T6 aluminium, per EN 10.346:2009 (DX51D+Z). Previously degreased visible metal components electrostatically painted with polyester or epoxy powder and polymerised at 200ºC. Paint thickness of 50/60 micrometres and colour tolerance per DIN 5033. Floor-to-ceiling single-glazed door leaf, with an 8mm-thick tempered glass pane framed by painted aluminium profiles, which is 1,058 mm wide and 40 mm thick. Hardware: dampened rollers with stoppers in the track, vertical short handle or vertical long handle with lock. Re-installation and interchangeability of modules. Quality assurance EN ISO 9001:2008 certified by LLOYD’S. Includes all parts necessary for installation. Supplied and installed by manufacturer-approved distributor. |
| --- | --- |

| ### MODULE G Clarit outer sliding door | M2. Supply and installation of LINE demountable partition with hidden profiling, module G: glazed outer sliding door with floor-to-ceiling door jambs. Standard module width: 1,058 mm. Total thickness: 133 mm, with the door leaf gliding along a track fixed to the ceiling. Made with a self-supporting inner frame in Sendzimir galvanised steel and outer frame in 6060-T6 aluminium, per EN 10.346:2009 (DX51D+Z). Previously degreased visible metal components electrostatically painted with polyester or epoxy powder and polymerised at 200ºC. Paint thickness of 50/60 micrometres and colour tolerance per DIN 5033. Floor-to-ceiling tempered glazed Clarit door leaf, 1,058 mm wide and 10 mm thick. Hardware: dampened rollers with stoppers in the track, vertical short handle or vertical long handle with lock. Re-installation and interchangeability of modules. Quality assurance EN ISO 9001:2008 certified by LLOYD’S. Includes all parts necessary for installation. Supplied and installed by manufacturer-approved distributor. |
| --- | --- |

| ### MODULE W Blind outer sliding door | M2. Supply and installation of LINE demountable partition with hidden profiling, module W: blind double outer sliding door with floor-to-ceiling door jambs. Standard module width: 2,116 mm. Total thickness: 148 mm, with the door leaves gliding along a track fixed to the ceiling. Made with a self-supporting inner frame in Sendzimir galvanised steel and outer frame in 6060-T6 aluminium, per EN 10.346:2009 (DX51D+Z). Previously degreased visible metal components electrostatically painted with polyester or epoxy powder and polymerised at 200ºC. Paint thickness of 50/60 micrometres and colour tolerance per DIN 5033. Floor-to-ceiling door leaves made in particle-board panels laminated with semi-rigid vinyl coatings. Other surface finish options include fine hardwoods or melamine and lacquered MDF panels. Door leaves are 1,058 mm wide and 40 mm thick. Hardware: dampened rollers with stoppers in the track, vertical short handles or vertical long handles with lock. Re-installation and interchangeability of modules. Quality assurance EN ISO 9001:2008 certified by LLOYD’S. Includes all parts necessary for installation. Supplied and installed by manufacturer-approved distributor. |
| --- | --- |

| ### MODULE W Framed glazed outer sliding door | M2. Supply and installation of 1:10 demountable partition with hidden profiling, module W: framed glazed outer double sliding door with floor-to-ceiling door jambs. Standard module width: 2,116 mm. Total thickness: 148 mm, with the door leaves gliding along a track fixed to the ceiling. Made with a self-supporting inner frame in Sendzimir galvanised steel and outer frame in 6060-T6 aluminium, per EN 10.346:2009 (DX51D+Z). Previously degreased visible metal components electrostatically painted with polyester or epoxy powder and polymerised at 200ºC. Paint thickness of 50/60 micrometres and colour tolerance per DIN 5033. Floor-to-ceiling single-glazed door leaves, with an 8 mm-thick tempered glass panes framed by painted aluminium profiles, which are 1,058 mm wide and 40 mm thick. Hardware: dampened rollers with stoppers in the track, vertical short handles or vertical long handles with lock. Re-installation and interchangeability of modules. Quality assurance EN ISO 9001:2008 certified by LLOYD’S. Includes all parts necessary for installation. Supplied and installed by manufacturer-approved distributor. |
| --- | --- |

| ### MODULE W Clarit outer sliding door | M2. Supply and installation of LINE demountable partition with hidden profiling, module G: glazed outer double sliding door with floor-to-ceiling door jambs. Standard module width: 2,116 mm. Total thickness: 133 mm, with the door leaves gliding along a track fixed to the ceiling. Made with a self-supporting inner frame in Sendzimir galvanised steel and outer frame in 6060-T6 aluminium, per EN 10.346:2009 (DX51D+Z). Previously degreased visible metal components electrostatically painted with polyester or epoxy powder and polymerised at 200ºC. Paint thickness of 50/60 micrometres and colour tolerance per DIN 5033. Floor-to-ceiling tempered glazed Clarit door leaves, 1,058 mm wide and 10 mm thick. Hardware: dampened rollers with stoppers in the track, vertical short handles or vertical long handles with lock. Re-installation and interchangeability of modules. Quality assurance EN ISO 9001:2008 certified by LLOYD’S. Includes all parts necessary for installation. Supplied and installed by manufacturer-approved distributor. |
| --- | --- |