D82/D92

WALL COVER

Product file

| ### D82 | M2. Supply and installation of D82 wall cover with visible profiling: totally blind. Standard module width: 1,200 mm. Total thickness: 33 mm. Covers walls, columns and vertical ducts. Telescopic system to compensate for uneven wall surfaces up to 118 mm. Made with Sendzimir galvanised-steel inner and outer frames per EN 10.142-A1:1994 (DX51D+Z). Previously degreased visible metal components electrostatically painted with polyester or epoxy powder and polymerised at 200ºC. Total paint thickness of 50/60 micrometres and colour tolerance per DIN 5033. Same system as M82 partitioning: 13-mm plasterboard panels coated with 0.3-mm vinyl laminate. Interior insulation with 50-mm thick, 30/40 Kg/m3 dense mineral fibre. Components for window, door and corner joins. Re-installation and interchangeability of modules. Quality assurance EN ISO 9001: 2008 certified by LLOYD’S. Includes all parts necessary for installation. Supplied and installed by manufacturer-approved distributor. |
| --- | --- |

| ### D92 | M2. Supply and installation of D92 wall cover with hidden profiling: totally blind. Standard module width: 1,200 mm. Total thickness: 33 mm. Covers walls, columns and vertical ducts. Telescopic system to compensate for uneven wall surfaces up to 118 mm. Made with Sendzimir galvanised-steel inner and outer frames per EN 10.142-A1:1994 (DX51D+Z). Previously degreased visible metal components electrostatically painted with polyester or epoxy powder and polymerised at 200ºC. Total paint thickness of 50/60 micrometres and colour tolerance per DIN 5033. Same system as M92 partitioning: 13-mm hardboard panels with 2-mm PVC edging coated with 0.3-mm vinyl laminate. The grooves the modulation features house a 10-mm wide, PVC splice joint. Interior insulation with 50-mm thick, 30/40 Kg/m3 dense mineral fibre. Components for window, door and corner joins. Re-installation and interchangeability of modules. Quality assurance EN ISO 9001: 2008 certified by LLOYD’S. Includes all parts necessary for installation. Supplied and installed by manufacturer-approved distributor. |
| --- | --- |