M92 DEMOUNTABLE  PARTITION

Product file

| ### MODULE A | M2. Supply and installation of M92 demountable partition with hidden profiling, module A: totally blind. Standard module width: 1,200 mm. Total thickness: 80 mm. Made with Sendzimir galvanised-steel inner and outer frames per EN 10.142-A1:1994 (DX51D+Z). Previously degreased visible metal components electrostatically painted with polyester or epoxy powder and polymerised at 200ºC. Paint thickness of 50/60 micrometres and colour tolerance per DIN 5033. 13-mm hardboard panels coated with 0.3-mm vinyl laminate and edged with 2-mm PVC. The grooves the modulation features house a 10-mm wide, PVC splice joint. Interior insulation with 50-mm thick, 30/40 Kg/m3 dense mineral fibre. Acoustic insulation: 41.3 dB per UNE 74040. Re-installation and interchangeability of modules. Quality assurance EN ISO 9001: 2008 certified by LLOYD’S. Includes all parts necessary for installation. Supplied and installed by manufacturer-approved distributor. |
| --- | --- |

| ### MODULE B | M2. Supply and installation of M92 demountable partition with hidden profiling, module B: mixed, double-glazed, central-glass section (blind to 882 mm and double-glazed to 2,082 mm). Standard module width: 1,200 mm. Total thickness: 80 mm. Made with a self-supporting, inner frame and an outer frame in Sendzimir galvanised steel per EN 10.142-A1:1994 (DX51D+Z). Previously degreased visible metal components electrostatically painted with polyester or epoxy powder and polymerised at 200ºC. Paint thickness of 50/60 micrometres and colour tolerance per DIN 5033. 13-mm hardboard panels coated with 0.3-mm vinyl laminate and edged with 2-mm PVC. The grooves the modulation features house a 10-mm wide, PVC splice joint. Interior insulation with 50-mm thick, 30/40 Kg/m3 dense mineral fibre. 4- to 6-mm thick panes. 52-mm gap between panes. The panes can be demounted from the frame independently thanks to the perimeter glass-stops on both faces. Re-installation and interchangeability of modules. Quality assurance EN ISO 9001: 2008 certified by LLOYD’S. Includes all parts necessary for installation. Supplied and installed by manufacturer-approved distributor. |
| --- | --- |

| ### MODULE C | M2. Supply and installation of M92 demountable partition with hidden profiling, module C: mixed, double-glazed, lower-glass section (glazed to 2,082 mm and blind to ceiling). Standard module width: 1,200 mm. Total thickness: 80 mm. Made with Sendzimir galvanised-steel inner and outer frames per EN 10.142-A1:1994 (DX51D+Z). Previously degreased visible metal components electrostatically painted with polyester or epoxy powder and polymerised at 200ºC. Paint thickness of 50/60 micrometres and colour tolerance per DIN 5033. 13-mm hardboard panels coated with 0.3-mm vinyl laminate and edged with 2-mm PVC. The grooves the modulation features house a 10-mm wide, PVC splice joint. Interior insulation with 50-mm thick, 30/40 Kg/m3 dense mineral fibre. 4- to 6-mm thick panes. 52-mm gap between panes. The panes can be demounted from the frame independently thanks to the perimeter glass-stops on both faces. Re-installation and interchangeability of modules. Quality assurance EN ISO 9001: 2008 certified by LLOYD’S. Includes all parts necessary for installation. Supplied and installed by manufacturer-approved distributor. |
| --- | --- |

| ### MODULE D | M2. Supply and installation of M92 demountable partition with hidden profiling, module D: totally double-glazed module. Standard module width: 1,200 mm. Total thickness: 80 mm. Made with Sendzimir galvanised-steel inner and outer frames per EN 10.142-A1:1994 (DX51D+Z). Previously degreased visible metal components electrostatically painted with polyester or epoxy powder and polymerised at 200ºC. Paint thickness of 50/60 micrometres and colour tolerance per DIN 5033. 4- to 6-mm thick panes. 52-mm gap between panes. The panes can be demounted from the frame independently thanks to the perimeter glass-stops on both faces. Re-installation and interchangeability of modules. Quality assurance EN ISO 9001: 2008 certified by LLOYD’S. Includes all parts necessary for installation. Supplied and installed by manufacturer-approved distributor. |
| --- | --- |

| ### MODULE E | M2. Supply and installation of M92 demountable partition with hidden profiling, module E: mixed, upper-glass section (blind to 882 mm and glazed to ceiling). Standard module width: 1,200 mm. Total thickness: 80 mm. Made with Sendzimir galvanised-steel inner and outer frames per EN 10.142-A1:1994 (DX51D+Z). Previously degreased visible metal components electrostatically painted with polyester or epoxy powder and polymerised at 200ºC. Paint thickness of 50/60 micrometres and colour tolerance per DIN 5033. 13-mm hardboard panels coated with 0.3-mm vinyl laminate and edged with 2-mm PVC. The grooves the modulation features house a 10-mm wide, PVC splice joint. Interior insulation with 50-mm thick, 30/40 Kg/m3 dense mineral fibre. 4- to 6-mm thick panes. 52-mm gap between panes. The panes can be demounted from the frame independently thanks to the perimeter glass-stops on both faces. Re-installation and interchangeability of modules. Quality assurance EN ISO 9001: 2008 certified by LLOYD’S. Includes all parts necessary for installation. Supplied and installed by manufacturer-approved distributor. |
| --- | --- |

| ### MODULE F | M2. Supply and installation of M92 demountable partition with hidden profiling, module F: mixed, fanlight (blind to 2,082 mm and glazed to ceiling). Standard module width: 1,200 mm. Total thickness: 80 mm. Made with Sendzimir galvanised-steel inner and outer frames per EN 10.142-A1:1994 (DX51D+Z). Previously degreased visible metal components electrostatically painted with polyester or epoxy powder and polymerised at 200ºC. Paint thickness of 50/60 micrometres and colour tolerance per DIN 5033. 13-mm hardboard panels coated with 0.3-mm vinyl laminate and edged with 2-mm PVC. The grooves the modulation features house a 10-mm wide, PVC splice joint. Interior insulation with 50/60-mm thick, 30/40 Kg/m3 dense mineral fibre. 4- to 6-mm thick panes. 52-mm gap between panes. The panes can be demounted from the frame independently thanks to the perimeter glass-stops on both faces. Re-installation and interchangeability of modules. Quality assurance EN ISO 9001: 2008 certified by LLOYD’S. Includes all parts necessary for installation. Supplied and installed by manufacturer-approved distributor. |
| --- | --- |

| ### MODULE J | M2. Supply and installation of M92 demountable partition with hidden profiling, module J: with blind or glazed door with blind top panel. Standard module width: 1,200 mm. Total thickness: 80 mm. Made with Sendzimir galvanised-steel inner and outer frames per EN 10.142A1:1994 (DX51D+Z). Previously degreased visible metal components electrostatically painted with polyester or epoxy powder and polymerised at 200ºC. Paint thickness of 50/60 micrometres and colour tolerance per DIN 5033. 13-mm hardboard panels coated with 0.3-mm vinyl laminate and edged with 2-mm PVC. The grooves the modulation features house a 10-mm wide, PVC splice joint. Interior insulation with 50/60-mm thick, 30/40 Kg/m3 dense mineral fibre. 830 x 2,040 x 40-mm solid hardboard door leaf coated with 0.3-mm vinyl laminate and edged with 2-mm PVC. “Meroni”-style door knob system with catch. Re-installation and interchangeability of modules. Quality assurance EN ISO 9001: 2008 certified by LLOYD’S. Includes all parts necessary for installation. Supplied and installed by manufacturer-approved distributor. |
| --- | --- |

| ### MODULE K | M2. Supply and installation of M92 demountable partition with hidden profiling, module K: with blind or glazed door with blind top panel. Standard module width: 909 mm. Total thickness: 80 mm. Made with Sendzimir galvanised-steel inner and outer frames per EN 10.142-A1:1994 (DX51D+Z). Previously degreased visible metal components electrostatically painted with polyester or epoxy powder and polymerised at 200ºC. Paint thickness of 50/60 micrometres and colour tolerance per DIN 5033. 13-mm hardboard panels coated with 0.3-mm vinyl laminate and edged with 2-mm PVC. The grooves the modulation features house a 10-mm wide, PVC splice joint. Interior insulation with 50/60-mm thick, 30/40 Kg/m3 dense mineral fibre. 830 x 2,040 x 40-mm solid hardboard door leaf coated with 0.3-mm vinyl laminate and edged with 2-mm PVC. “Meroni”-style door knob system with catch. Re-installation and interchangeability of modules. Quality assurance EN ISO 9001: 2008 certified by LLOYD’S. Includes all parts necessary for installation. Supplied and installed by manufacturer-approved distributor. |
| --- | --- |

| ### MODULE L | M2. Supply and installation of M92 demountable partition with hidden profiling, module L: with blind or glazed door with double-glazed side section and blind top panel. Standard module width: 1,200 mm. Total thickness: 80 mm. Made with Sendzimir galvanised-steel inner and outer frames per EN 10.142-A1:1994 (DX51D+Z). Previously degreased visible metal components electrostatically painted with polyester or epoxy powder and polymerised at 200ºC. Paint thickness of 50/60 micrometres and colour tolerance per DIN 5033. 13-mm hardboard panels coated with 0.3-mm vinyl laminate and edged with 2-mm PVC. The grooves the modulation features house a 10-mm wide, PVC splice joint. Interior insulation with 50-mm thick, 30/40 Kg/m3 dense mineral fibre. 4- to 6-mm thick panes. 52-mm gap between panes. The panes can be demounted from the frame independently thanks to the perimeter glass-stops on both faces. 830 x 2,040 x 40-mm solid hardboard door leaf coated with 0.3-mm vinyl laminate and edged with 2-mm PVC. “Meroni”-style door knob system with catch. Re-installation and interchangeability of modules. Quality assurance EN ISO 9001: 2008 certified by LLOYD’S. Includes all parts necessary for installation. Supplied and installed by manufacturer-approved distributor. |
| --- | --- |

| ### MODULE M | M2. Supply and installation of M92 demountable partition with hidden profiling, module M: with blind door with stiles to ceiling and upper breast. Standard module width: 909 mm. Total thickness: 80 mm. Made with Sendzimir galvanised-steel inner and outer frames per EN 10.142-A1:1994 (DX51D+Z). Previously degreased visible metal components electrostatically painted with polyester or epoxy powder and polymerised at 200ºC. Paint thickness of 50/60 micrometres and colour tolerance per DIN 5033. 13-mm hardboard panels coated with 0.3-mm vinyl laminate and edged with 2-mm PVC. The grooves the modulation features house a 10-mm wide, PVC splice joint. Interior insulation with 50-mm thick, 30/40 Kg/m3 dense mineral fibre. 830 x 2,040 x 40-mm solid hardboard door leaf coated with 0.3-mm vinyl laminate and edged with 2-mm PVC included. “Meroni”-style door knob system with catch. Re-installation and interchangeability of modules. Quality assurance EN ISO 9001: 2008 certified by LLOYD’S. Includes all parts necessary for installation. Supplied and installed by manufacturer-approved distributor. |
| --- | --- |

| ### MODULE O | M2. Supply and installation of M92 demountable partition with hidden profiling, module O: with blind or glazed door with double-glazed fanlight. Standard module width: 1,200 mm. Total thickness: 80 mm. Made with Sendzimir galvanised-steel inner and outer frames per EN 10.142-A1:1994 (DX51D+Z). Previously degreased visible metal components electrostatically painted with polyester or epoxy powder and polymerised at 200ºC. Paint thickness of 50/60 micrometres and colour tolerance per DIN 5033. 13-mm hardboard panels coated with 0.3-mm vinyl laminate and edged with 2-mm PVC. The grooves the modulation features house a 10-mm wide, PVC splice joint. Interior insulation with 50-mm thick, 30/40 Kg/m3 dense mineral fibre. 4- to 6-mm thick panes. 52-mm gap between panes. The panes can be demounted from the frame independently thanks to the perimeter glass-stops on both faces. 830 x 2,040 x 40-mm solid hardboard door leaf coated with 0.3-mm vinyl laminate and edged with 2-mm PVC. “Meroni”-style door knob system with catch. Re-installation and interchangeability of modules. Quality assurance EN ISO 9001: 2008 certified by LLOYD’S. Includes all parts necessary for installation. Supplied and installed by manufacturer-approved distributor. |
| --- | --- |

| ### MODULE P | M2. Supply and installation of M92 demountable partition with hidden profiling, module P: with blind or glazed door with double-glazed fanlight. Standard module width: 909 mm. Total thickness: 80 mm. Made with Sendzimir galvanised-steel inner and outer frames per EN 10.142-A1:1994 (DX51D+Z). Previously degreased visible metal components electrostatically painted with polyester or epoxy powder and polymerised at 200ºC. Paint thickness of 50/60 micrometres and colour tolerance per DIN 5033. The grooves the modulation features house a 10-mm wide, PVC splice joint. 4- to 6-mm thick panes. 52-mm gap between panes. The panes can be demounted from the frame independently thanks to the perimeter glass-stops on both faces. 830 x 2,040 x 40-mm solid hardboard door leaf coated with 0.3-mm vinyl laminate and edged with 2-mm PVC. “Meroni”-style door knob system with catch. Re-installation and interchangeability of modules. Quality assurance EN ISO 9001: 2008 certified by LLOYD’S. Includes all parts necessary for installation. Supplied and installed by manufacturer-approved distributor. |
| --- | --- |

| ### MODULE Q | M2. Supply and installation of M92 demountable partition with hidden profiling, module Q: with blind or glazed door with double-glazed side section and fanlight. Standard module width: 1,200 mm. Total thickness: 80 mm. Made with Sendzimir galvanised-steel inner and outer frames per EN 10.142-A1:1994 (DX51D+Z). Previously degreased visible metal components electrostatically painted with polyester or epoxy powder and polymerised at 200ºC. Paint thickness of 50/60 micrometres and colour tolerance per DIN 5033. The grooves the modulation features house a 10-mm wide, PVC splice joint. 4- to 6-mm thick panes. 52-mm gap between panes. The panes can be demounted from the frame independently thanks to the perimeter glass-stops on both faces. 830 x 2,040 x 40-mm solid hardboard door leaf coated with 0.3-mm vinyl laminate and edged with 2-mm PVC. “Meroni”-style door knob system with catch. Re-installation and interchangeability of modules. Quality assurance EN ISO 9001: 2008 certified by LLOYD’S. Includes all parts necessary for installation. Supplied and installed by manufacturer-approved distributor. |
| --- | --- |

| ### MODULE R | M2. Supply and installation of M92 demountable partition with hidden profiling, module R: with blind double doors with blind top panel. Standard module width: 2,400 mm. Total thickness: 80 mm. Made with Sendzimir galvanised-steel inner and outer frames per EN 10.142-A1:1994 (DX51D+Z). Previously degreased visible metal components electrostatically painted with polyester or epoxy powder and polymerised at 200ºC. Paint thickness of 50/60 micrometres and colour tolerance per DIN 5033. 13-mm hardboard panels coated with 0.3-mm vinyl laminate and edged with 2-mm PVC. The grooves the modulation features house a 10-mm wide, PVC splice joint. Interior insulation with 50-mm thick, 30/40 Kg/m3 dense mineral fibre. 830 x 2,040 x 40-mm solid hardboard door leaves coated with 0.3-mm vinyl laminate and edged with 2-mm PVC. “Meroni”-style door knob system with catch. Re-installation and interchangeability of modules. Quality assurance EN ISO 9001: 2008 certified by LLOYD’S. Includes all parts necessary for installation. Supplied and installed by manufacturer-approved distributor. |
| --- | --- |

| ### MODULE S | M2. Supply and installation of M92 demountable partition with hidden profiling, module S: with blind double doors with blind top panel. Standard module width: 1,742 mm. Total thickness: 80 mm. Made with Sendzimir galvanised-steel inner and outer frames per EN 10.142-A1:1994 (DX51D+Z). Previously degreased visible metal components electrostatically painted with polyester or epoxy powder and polymerised at 200ºC. Paint thickness of 50/60 micrometres and colour tolerance per DIN 5033. 13-mm hardboard panels coated with 0.3-mm vinyl laminate and edged with 2-mm PVC. The grooves the modulation features house a 10-mm wide, PVC splice joint. Interior insulation with 50-mm thick, 30/40 Kg/m3 dense mineral fibre. 830 x 2,040 x 40-mm solid hardboard door leaves coated with 0.3-mm vinyl laminate and edged with 2-mm PVC. “Meroni”-style door knob system with catch. Re-installation and interchangeability of modules. Quality assurance EN ISO 9001: 2008 certified by LLOYD’S. Includes all parts necessary for installation. Supplied and installed by manufacturer-approved distributor. |
| --- | --- |

| ### MODULE T | M2. Supply and installation of M92 demountable partition with hidden profiling, module T: with blind double doors with stiles to ceiling and upper breast. Standard module width: 1,742 mm. Total thickness: 80 mm. Made with Sendzimir galvanised-steel inner and outer frames per EN 10.142-A1:1994 (DX51D+Z). Previously degreased visible metal components electrostatically painted with polyester or epoxy powder and polymerised at 200ºC. Paint thickness of 50/60 micrometres and colour tolerance per DIN 5033. 13-mm hardboard panels coated with 0.3-mm vinyl laminate and edged with 2-mm PVC. The grooves the modulation features house a 10-mm wide, PVC splice joint. Interior insulation with 50-mm thick, 30/40 Kg/m3 dense mineral fibre. 830 x 2,040 x 40-mm solid hardboard door leaves with 1,660 x 600 x 40-mm upper breast coated with 0.3-mm vinyl laminate and edged with 2-mm PVC. “Meroni”-style door knob system with catch. Re-installation and interchangeability of modules. Quality assurance EN ISO 9001: 2008 certified by LLOYD’S. Includes all parts necessary for installation. Supplied and installed by manufacturer-approved distributor. |
| --- | --- |

| ### MODULE U | M2. Supply and installation of M92 demountable partition with hidden profiling, module U: with blind double doors with blind top panel. Standard module width: 1,200 mm. Total thickness: 80 mm. Made with Sendzimir galvanised-steel inner and outer frames per EN 10.142-A1:1994 (DX51D+Z). Previously degreased visible metal components electrostatically painted with polyester or epoxy powder and polymerised at 200ºC. Paint thickness of 50/60 micrometres and colour tolerance per DIN 5033. 13-mm hardboard panels coated with 0.3-mm vinyl laminate and edged with 2-mm PVC. The grooves the modulation features house a 10-mm wide, PVC splice joint. Interior insulation with 50-mm thick, 30/40 Kg/m3 dense mineral fibre. 830 x 2,040 x 40-mm and 291 x 2,040 x 40-mm solid hardboard door leaves coated with 0.3-mm vinyl laminate and edged with 2-mm PVC. “Meroni”-style door knob system with catch. Re-installation and interchangeability of modules. Quality assurance EN ISO 9001: 2008 certified by LLOYD’S. Includes all parts necessary for installation. Supplied and installed by manufacturer-approved distributor. |
| --- | --- |

| ### MODULE V | M2. Supply and installation of M92 demountable partition with hidden profiling, module V: blind double doors with double-glazed fanlight. Standard module width: 1,200 mm. Total thickness: 80 mm. Made with Sendzimir galvanised-steel inner and outer frames per EN 10.142-A1:1994 (DX51D+Z). Previously degreased visible metal components electrostatically painted with polyester or epoxy powder and polymerised at 200ºC. Paint thickness of 50/60 micrometres and colour tolerance per DIN 5033. 13-mm hardboard panels coated with 0.3-mm vinyl laminate and edged with 2-mm PVC. The grooves the modulation features house a 10-mm wide, PVC splice joint. Interior insulation with 50-mm thick, 30/40 Kg/m3 dense mineral fibre. 4- to 6-mm thick panes. 52-mm gap between panes. The panes can be demounted from the frame independently thanks to the perimeter glass-stops on both faces. 830 x 2,040 x 40-mm and 291 x 2,040 x 40-mm solid hardboard door leaves coated with 0.3-mm vinyl laminate and edged with 2-mm PVC. “Meroni”-style door knob system with catch. Re-installation and interchangeability of modules. Quality assurance EN ISO 9001: 2008 certified by LLOYD’S. Includes all parts necessary for installation. Supplied and installed by manufacturer-approved distributor. |
| --- | --- |

| ### MODULE Y | M2. Supply and installation of M92 demountable partition with hidden profiling, module Y: with blind double doors with double-glazed fanlight. Standard module width: 2,400 mm. Total thickness: 80 mm. Made with Sendzimir galvanised-steel inner and outer frames per EN 10.142-A1:1994 (DX51D+Z). Previously degreased visible metal components electrostatically painted with polyester or epoxy powder and polymerised at 200ºC. Paint thickness of 50/60 micrometres and colour tolerance per DIN 5033. 13-mm hardboard panels coated with 0.3-mm vinyl laminate and edged with 2-mm PVC. The grooves the modulation features house a 10-mm wide, PVC splice joint. Interior insulation with 50-mm thick, 30/40 Kg/m3 dense mineral fibre. 4- to 6-mm thick panes. 52-mm gap between panes. The panes can be demounted from the frame independently thanks to the perimeter glass-stops on both faces. 830 x 2,040 x 40-mm solid hardboard door leaves coated with 0.3-mm vinyl laminate and edged with 2-mm PVC. “Meroni”-style door knob system with catch. Re-installation and interchangeability of modules. Quality assurance EN ISO 9001: 2008 certified by LLOYD’S. Includes all parts necessary for installation. Supplied and installed by manufacturer-approved distributor. |
| --- | --- |

| ### MODULE Z | M2. Supply and installation of M92 demountable partition with hidden profiling, module Z: with blind double doors with double-glazed fanlight. Standard module width: 1,742 mm. Total thickness: 80 mm. Made with Sendzimir galvanised-steel inner and outer frames per EN 10.142-A1:1994 (DX51D+Z). Previously degreased visible metal components electrostatically painted with polyester or epoxy powder and polymerised at 200ºC. Paint thickness of 50/60 micrometres and colour tolerance per DIN 5033. 4- to 6-mm thick panes. 52-mm gap between panes. The panes can be demounted from the frame independently thanks to the perimeter glass-stops on both faces. 830 x 2,040 x 40-mm solid hardboard door leaves coated with 0.3-mm vinyl laminate and edged with 2-mm PVC. “Meroni”-style door knob system with catch. Re-installation and interchangeability of modules. Quality assurance EN ISO 9001: 2008 certified by LLOYD’S. Includes all parts necessary for installation. Supplied and installed by manufacturer-approved distributor. |
| --- | --- |